MIRA RING

Supplier Summary & Validation Report

━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━

Date: April 2026

Scope: 16 supplier listings sourced from 1688.com and Alibaba.com

Validated Against: Mira Ring device requirements specification

Classification: Confidential — Client Use Only

Table of Contents

1. Mira Ring Key Requirements

2. Supplier Profiles (1–16)

3. Validation Summary Table

4. Priority Recommendations

1. Mira Ring Key Requirements

The following specifications define the target requirements for the Mira Ring device. All suppliers are evaluated against these criteria.

| Requirement | Specification |
| --- | --- |
| Touch Sensing | Capacitive touch outer surface |
| Connectivity | BLE 5.x |
| Water Resistance | 5ATM (50m) minimum |
| Charging | Magnetic (no exposed port) |
| Body Material | Stainless steel or equivalent — matte black |
| Sizes | Multiple standard ring sizes |
| Battery | LiPo, all-day use |
| Certifications | FCC, CE, RoHS, REACH |
| Ex-Factory Target | Below $42.50 (¥~305 at current rate) |
| Form Factor | Wide flat band, concealed electronics, fashion ring appearance |

2. Supplier Profiles

Supplier 1 — Shenzhen Huagepeng Technology Co., Ltd.

Chinese Name: 深圳市华鸽朋科技有限公司

Location: Shenzhen, Guangdong

Platform: 1688.com

Type: Factory wholesale

Product: Equantu SQ976 — Touch Screen Counter Smart Ring

Category: Prayer / tally counter ring

| Specification | Value |
| --- | --- |
| Price | ¥80–95/unit |
| MOQ | 3 units |
| Battery | 40 mAh |
| Sizes | 18 / 20 / 22 mm inner diameter |
| Colors | Black, White |
| Water Resistance | Not specified |
| Charging | Data cable (not magnetic) |
| BLE | Yes (version unspecified) |
| Certifications | None listed |
| Touch | Touchscreen display (counter) |
| Health Sensors | None |

Mira Ring Validation

✗ Counter ring only — no gesture control for glasses interaction

✗ No 5ATM / no waterproof rating

✗ No magnetic charging

✗ No health sensors

✗ Plastic body (no stainless steel)

✓ Bluetooth, multiple sizes, black color

Verdict: Does not meet Mira Ring requirements.

Supplier 2 — Shenzhen Hangu Cloud Technology Co., Ltd.

Chinese Name: 深圳市汉谷云科技有限公司

Location: Shenzhen, Guangdong

Platform: 1688.com

Type: Factory direct sales (multi-model reseller)

Product: Equantu multi-model counter rings (QB702, QB702Lite, QB708, QB709, QB809, SQ668, SQ976)

Category: Prayer / tally counter ring

| Specification | Value |
| --- | --- |
| Price | ¥45–290/unit (model dependent) |
| MOQ | 3 units |
| Battery | 18–55 mAh (model dependent) |
| Sizes | 18 / 20 / 22 mm |
| Materials | Plastic, zinc alloy, ceramic, titanium alloy, stainless steel + gel resin |
| Colors | Black, Silver, Gold |
| Water Resistance | "Waterproof" (no IP/ATM rating) |
| Charging | Data cable / charging case (varies by model) |
| BLE | Yes (version unspecified) |
| Certifications | None listed |
| Touch | Touchscreen (counter) |
| Health Sensors | None |

Mira Ring Validation

✗ Counter ring only — no gesture control for glasses interaction

✗ No confirmed 5ATM rating

✗ No magnetic charging

✗ No health sensors

⚠ Some models have premium materials (ceramic QB709, zinc alloy QB702)

✓ Wide material variety, BLE, multiple sizes

Verdict: Does not meet Mira Ring requirements.

Supplier 3 — Shenzhen Weitecheng Technology Co., Ltd.

Chinese Name: 深圳市韦特诚科技有限公司

Location: Shenzhen, Guangdong

Platform: 1688.com

Type: Manufacturer

Sales Volume: 500+

Product: Equantu SQ9761/SQ9762 — Smart Counter Ring (OLED display, replaceable ring holders)

Category: Prayer / tally counter ring

| Specification | Value |
| --- | --- |
| Price | ¥95–110/unit |
| MOQ | 1 unit |
| Battery | 40 mAh |
| Sizes | 18 / 20 / 22 mm (replaceable holders) |
| Colors | Black, White, Dark Violet, Green, Blue, Pink |
| Water Resistance | Not specified |
| Charging | USB data cable |
| BLE | Yes |
| Certifications | None listed |
| Touch | Hand press button |
| Health Sensors | None |
| Private Label | Available |

Mira Ring Validation

✗ Counter ring — hand press button, not capacitive touch

✗ No 5ATM

✗ No magnetic charging

✗ No health sensors, plastic body

✓ Low MOQ, private label available

Verdict: Does not meet Mira Ring requirements.

Supplier 4 — Shenzhen Zhentongxin Technology Co., Ltd.

Chinese Name: 深圳市振通鑫科技有限公司

Location: Shenzhen, Guangdong

Platform: 1688.com

Type: Distributor/reseller

Product: Equantu SQ976 — Cross-border bestseller counter ring

Category: Prayer / tally counter ring

| Specification | Value |
| --- | --- |
| Price | ¥85–90/unit |
| MOQ | 1 unit |
| Battery | 40 mAh |
| Sizes | 18 / 20 / 22 mm |
| Colors | Black, White |
| Water Resistance | Not specified |
| Charging | Data cable |
| BLE | Yes |
| Certifications | None listed |
| Touch | Touchscreen counter |
| Health Sensors | None |
| Private Label | Available |
| Dropshipping | Supported |

Mira Ring Validation

✗ Same SQ976 counter ring as Supplier 1 — no gesture control

✗ No 5ATM, no magnetic charging, no health sensors

✓ Low MOQ of 1, private label available

Verdict: Does not meet Mira Ring requirements.

Supplier 5 — Shenzhen Beiqi Information Technology Co., Ltd.

Chinese Name: 深圳市贝奇信息技术有限公司

Location: Shenzhen, Guangdong

Platform: 1688.com

Type: Factory wholesale

Sales Volume: 4,300+

Product: BeiQi S7 — OLED Counter Ring (multi-channel, luminous)

Category: Prayer / tally counter ring

| Specification | Value |
| --- | --- |
| Price | ¥38–46/unit |
| MOQ | 10 units |
| Battery | Not specified |
| Sizes | Ring (finger wear) |
| Colors | Black, Gray, Gold, Green |
| Water Resistance | "Waterproof" (no IP/ATM rating) |
| Charging | Not specified |
| BLE | Yes |
| Certifications | None listed |
| Touch | Touch + luminous |
| Health Sensors | None |

Mira Ring Validation

✗ Counter ring — multi-channel tally, not gesture control

✗ No confirmed waterproof rating, no magnetic charging

✗ No health sensors

✓ High sales volume (4,300+) indicates manufacturing reliability, low price

Verdict: Does not meet Mira Ring requirements.

Supplier 6 — Dongguan Xingyao Zhiqing Technology Co., Ltd.

Chinese Name: 东莞星耀智擎科技有限公司

Location: Foshan, Guangdong

Platform: 1688.com

Type: Distributor / possible factory

Sales Volume: 1,500+

Product: Equantu multi-model — Middle East smart rings (QB702, QB709, QB708, SQ668, QB702Lite, SQ9761)

Category: Prayer / social counter ring

| Specification | Value |
| --- | --- |
| Price | ¥49–308/unit (model dependent) |
| MOQ | 1 unit |
| Battery | 40 mAh |
| Sizes | 18 / 20 / 22 mm |
| Materials | Zinc alloy (QB702), Ceramic (QB709), plastic variants |
| Colors | Black, Silver, Gold |
| Water Resistance | Not specified |
| Charging | USB / charging case (model dependent) |
| BLE | Bluetooth 5.4 (noted in store ecosystem) |
| Certifications | CE (partial) |
| Touch | Touchscreen |
| Health Sensors | None |
| Dropshipping | Supported |
| Ships | Within 48 hours |

Mira Ring Validation

✗ Counter ring category — no gesture control for glasses interaction

✗ No 5ATM, no magnetic charging, no health sensors

⚠ BLE 5.4 noted (positive); CE partial certification

⚠ Premium material options (ceramic QB709 at ¥308) show capacity for high-end finishes

Verdict: Does not meet Mira Ring requirements.

Supplier 7 — Huizhou Xinyao Electronic Technology Co., Ltd.

Chinese Name: 惠州市鑫曜电子科技有限公司

Location: Huizhou, Guangdong

Platform: 1688.com

Type: Factory direct supply

Sales Volume: 14,000+ (highest among all 16 suppliers)

Product: Health Smart Ring — heart rate, blood oxygen, sleep monitoring, magnetic charging

Category: Health / fitness smart ring

| Specification | Value |
| --- | --- |
| Price | ¥47.88 (with magnetic cable) / ¥62.88 (with charging case) |
| MOQ | 1 unit |
| Battery | 18 mAh |
| Sizes | US ring sizes #6–#14 |
| Weight | 3.6–4.5 g (ring only) |
| Colors | Black, Silver, Gold |
| Water Resistance | "Waterproof" (no explicit IP/ATM rating) |
| Charging | Magnetic cable charger OR charging compartment |
| BLE | Yes (version unspecified) |
| Certifications | None listed |
| Touch | Not explicitly stated |
| Health Sensors | Heart rate, blood pressure, SpO2, sleep monitoring, step counting |
| Dropshipping | Supported |

Mira Ring Validation

✓ Magnetic charging confirmed (two options)

✓ Health sensors present (HR, SpO2, sleep)

✓ Very wide size range (#6–#14), lightweight (3.6–4.5g)

✓ Extremely high sales volume (14,000+) — proven manufacturing scale

✓ Low MOQ and price (¥47.88 well under target)

✗ No confirmed waterproof rating (5ATM not stated)

✗ No certifications (FCC, CE, RoHS not listed)

✗ No confirmed capacitive touch / gesture control

✗ BLE version unspecified

Verdict: Partial match. Strong on price, size range, and sales volume. Key gaps: waterproof rating, certifications, gesture/touch confirmation.

Supplier 8 — Shenzhen Xinzhiyan Medical Technology Co., Ltd.

Chinese Name: 深圳市新智研医疗科技有限公司

Location: Shenzhen, Guangdong

Platform: 1688.com

Type: Factory direct sales (medical technology company)

Sales Volume: 4,000+

Product: HK01 — Health Smart Ring (Amazon cross-border bestseller, AI health wearable)

Category: Medical-grade health smart ring

| Specification | Value |
| --- | --- |
| Price | ¥89/unit |
| MOQ | 10 units |
| Battery | 18 mAh |
| Model | HK01 |
| Sizes | US #7 (17.3mm) / #8 (18.1mm) / #9 (18.9mm) / #10 (19.8mm) / #11 (20.6mm) |
| Weight | 3.8 g (ring only) |
| Material | Stainless steel + epoxy |
| Colors | Space gray, Black, Silver, Purple, Gold, Rose Gold |
| Water Resistance | IP68 (confirmed) |
| Charging | Cradle / stand charger (NOT magnetic) |
| BLE | Bluetooth LE 5.0 (confirmed) |
| Certifications | CE (confirmed), FCC (compliance docs available) |
| Touch | Touchscreen mentioned |
| Health Sensors | Heart rate, SpO2, HRV, stress, female menstrual cycle, step counting, sleep |
| Sensor Chips | SC7A20H accelerometer; proprietary HR sensor |
| Dropshipping | Supported |
| Invoice | Provided |

Mira Ring Validation

✓ IP68 waterproofing (equivalent to or exceeding 5ATM)

✓ BLE 5.0 confirmed

✓ CE + FCC certifications confirmed

✓ Stainless steel body

✓ Comprehensive health sensors

✓ Medical tech company — highest credibility for quality and compliance

✓ 4,000+ sales, ¥89 well within target

✗ Charging is cradle stand, NOT magnetic (critical gap for Mira Ring)

✗ Matte black finish not confirmed

✗ Capacitive touch spec not explicitly confirmed

⚠ IP68 vs. 5ATM — technically similar but not identical nomenclature

Verdict: Strong match except charging method. Best-documented supplier with confirmed certs and BLE version. Worth engaging to ask if magnetic charging variant exists.

Supplier 9 — Huizhou Xinyao Electronic Technology Co., Ltd.

DUPLICATE of Supplier 7. Same company, same 1688 offer ID (831177742048), same pricing. Two separate HTML files appear to be scrapes of the same listing at different times or via different URL paths. Refer to Supplier 7 for all details.

Supplier 10 — Huaou Tonghui (Suzhou) Technology Co., Ltd.

Chinese Name: 华欧同惠科技有限公司

Location: Suzhou, Jiangsu (outside typical Shenzhen/Guangdong cluster)

Platform: 1688.com

Type: Manufacturer

Product: K80 — Touch Display Smart Ring (TikTok control, health monitoring)

Category: Health / lifestyle smart ring

| Specification | Value |
| --- | --- |
| Price | ¥119/unit (flat) |
| MOQ | 2 units |
| Battery | 15 mAh |
| Model | K80 |
| Sizes | US #7–#13 |
| Weight | 3.5 g |
| Colors | Gold, Silver, Black |
| Water Resistance | 5ATM (confirmed — 5ATM级防水 explicitly stated) |
| Charging | Charging case (not confirmed magnetic) |
| BLE | Bluetooth (version unspecified) |
| Certifications | None found |
| Touch | Touch screen design (触摸设计, confirmed) |
| Health Sensors | Sleep monitoring, heart rate, SpO2, step counting |
| Dropshipping | Supported |
| Invoice | Contact customer service |

Mira Ring Validation

✓ 5ATM water resistance confirmed (one of only two suppliers with this)

✓ Touchscreen design confirmed

✓ Black color option, lightweight (3.5g), sizes #7–#13

✓ Health sensors (HR, SpO2, sleep)

✓ Very low MOQ (2 units)

✗ Magnetic charging not confirmed (uses charging case)

✗ BLE version unspecified

✗ No certifications

⚠ Higher price (¥119) vs. competitors

⚠ Suzhou location may affect logistics vs. Shenzhen cluster

Verdict: Strong candidate. 5ATM confirmed, touch confirmed. Key follow-up: charging method (magnetic?), BLE version, certifications.

Supplier 11 — Guangzhou Xuanzhi Technology Co., Ltd.

Chinese Name: 广州玄致科技有限公司

Location: Guangzhou, Guangdong

Platform: 1688.com

Type: Manufacturer (factory link present)

Sales Volume: 2,000+

Product: Q8 / R08 — Health Smart Ring with magnetic charging ("Oura-level waterproof")

Category: Premium health smart ring

| Specification | Value |
| --- | --- |
| Price | ¥105/unit (≥5), ¥103/unit (≥100), ¥99/unit (≥2,000) |
| MOQ | 5 units |
| Battery | 15 mAh (sizes ≤#9), 18 mAh (sizes #10+) |
| Models | Q8 / R08 |
| Sizes | US standard sizes available |
| Weight | ~3.5 g (size #11) |
| Colors | Black, Gold |
| Water Resistance | "Oura-level waterproof" (no explicit IP/ATM number confirmed) |
| Charging | Magnetic wireless charging (磁吸充电, confirmed) |
| BLE | Bluetooth (version unspecified) |
| Certifications | None found |
| Touch | Touch ring (gesture control confirmed) |
| Health Sensors | HR, SpO2, HRV, stress, skin temperature, sleep tracking |
| Private Label | Available (authorized) |
| Invoice | Provided |
| Dropshipping | Supported |
| Nationwide Joint Guarantee | Yes |

Mira Ring Validation

✓ Magnetic charging confirmed

✓ Gesture/touch control confirmed

✓ Most comprehensive health sensor set (HR, SpO2, HRV, stress, skin temp, sleep)

✓ Private label available (important for Mira branding)

✓ Competitive volume pricing (¥99 at 2,000 units — well under $42.50 target)

✓ 2,000+ sales, invoice provided, nationwide guarantee

✗ No explicit waterproof rating — 'Oura-level' is marketing language, no IP/ATM number confirmed

✗ BLE version unspecified

✗ No certifications (FCC, CE, RoHS not listed)

Verdict: Strong candidate. Best combination of magnetic charging + gesture touch + health sensors + private label + volume pricing. Critical gap: waterproof rating needs numerical confirmation.

Supplier 12 — Shenzhen Migo E-commerce Co., Ltd.

Chinese Name: 深圳市迷购电子商务有限公司

Location: Shenzhen, Guangdong

Platform: 1688.com

Type: E-commerce company (possibly factory)

Sales Volume: 1,000+

Product: R11 — Touch Display Smart Ring with gesture control

Category: Health / lifestyle smart ring

| Specification | Value |
| --- | --- |
| Price | ¥103/unit |
| MOQ | 2 units |
| Battery | 15 mAh (sizes ≤#9), 18 mAh (sizes #10+) |
| Model | R11 |
| Material | Stainless steel |
| Colors | Not specified |
| Water Resistance | 1 ATM only (confirmed — far below 5ATM requirement) |
| Charging | Charging stand (not magnetic) |
| BLE | Bluetooth |
| Certifications | None found |
| Touch | Touch display, gesture control (photography, music, video) |
| Health Sensors | HR, SpO2, skin temperature, pressure, HRV, sleep |
| Dropshipping | Not supported |
| Invoice | None provided |

Mira Ring Validation

✗ 1ATM waterproofing — fails 5ATM requirement by a large margin

✗ Charging stand, not magnetic

✗ No certifications, no invoice

✓ Stainless steel body, gesture control, health sensors

✓ Low MOQ (2 units)

Verdict: Does not meet waterproofing requirement. Disqualified.

Supplier 13 — Shenzhen Huagepeng Technology Co., Ltd. (Alibaba.com)

Chinese Name: 深圳市华鸽朋科技有限公司 (Same company as Supplier 1, international platform)

Location: Guangdong, China

Platform: Alibaba.com (international)

Type: Manufacturer

Product: Equantu SQ976 — Zikr Smart Ring (Islamic prayer counter, 2026 model)

Category: Prayer / religious counter ring

| Specification | Value |
| --- | --- |
| Price | CN¥75–88/unit (100–2,000+ piece tiers) |
| MOQ | 100 pieces minimum |
| Battery Life | 5–10 days |
| Battery | 40 mAh |
| Model | SQ976 |
| Material | Plastic / Resin |
| Display | OLED |
| Water Resistance | IP67 (confirmed) |
| Charging | Magnetic charging (confirmed) |
| BLE | Bluetooth (version unspecified) |
| Certifications | FCC, RoHS, CE (all confirmed on Alibaba listing) |
| Touch | Touchscreen, App Control |
| Health Sensors | None — counter/alarm only |
| Private Mold | Yes |

Mira Ring Validation

✓ FCC, RoHS, CE certifications confirmed (most certs of any supplier)

✓ Magnetic charging confirmed

✓ IP67 waterproofing confirmed

✓ Same factory as Supplier 1 — established manufacturer

✗ Counter ring (SQ976) — not a health or gesture-control smart ring

✗ Plastic / resin body (not stainless steel)

✗ IP67, not 5ATM (close but not equivalent; IP67 = 1m submersion vs 5ATM = 50m)

✗ High MOQ (100 pieces minimum)

✗ No health sensors

Verdict: Certifications are valuable reference, but wrong product category. Useful as cert benchmark; not suitable as Mira Ring supplier without product pivot.

Supplier 14 — Shenzhen Mosslife Technology Co., Ltd.

Chinese Name: 深圳市莫斯莱夫科技有限公司

Location: Not confirmed

Platform: Alibaba.com (international)

Type: Manufacturer

Product: S7 — Digital Reading Smart Ring (counter, Type-C charging)

Category: Counter / social ring

| Specification | Value |
| --- | --- |
| Price | CN¥37–41/unit |
| MOQ | 1 piece |
| Battery Life | 11–30 days |
| Material | ABS plastic |
| Water Resistance | IPX8 (confirmed — full water immersion rated) |
| Charging | Type-C (not magnetic) |
| BLE | Bluetooth LE |
| Certifications | None listed |
| Touch | App control |
| Health Sensors | Social media notifications only |
| Private Mold | Yes |
| Ships | Within 3 days |

Mira Ring Validation

⚠ IPX8 is impressive for the price (full immersion), but rating system differs from 5ATM

✗ Type-C charging — no magnetic charging

✗ ABS plastic body (not stainless steel)

✗ Counter ring, no health sensors

✗ No certifications

✓ MOQ of 1, very low price (CN¥37–41), fast shipping

Verdict: Does not meet Mira Ring requirements. Wrong product category.

Supplier 15 — Hongsen Huanyu (Beijing) International Trading Co., Ltd.

Chinese Name: 宏森环宇（北京）国际贸易有限公司

Location: Beijing, China (not a Guangdong manufacturer)

Platform: Alibaba.com (international)

Type: Trading company (NOT a manufacturer)

Product: Couple's health smart ring — heart rate monitoring, ECG, LED display

Category: Health / fashion smart ring

| Specification | Value |
| --- | --- |
| Price | CN¥193–207/unit |
| MOQ | 10 pieces |
| Battery Life | Not specified |
| Material | ABS plastic |
| Colors | Deep Grey, Silver, Gold |
| Water Resistance | "Waterproof" (no IP/ATM rating) |
| Charging | "Other" (unspecified) |
| BLE | Bluetooth Smart Ring |
| Certifications | None listed |
| Touch | Touchscreen |
| Health Sensors | Heart rate, ECG (referenced in attributes), sleep |

Mira Ring Validation

✗ Beijing trading company — not a manufacturer; reselling Guangdong factory product at markup

✗ ABS plastic body

✗ No waterproof rating, no certifications, charging unspecified

✗ Highest price point (CN¥193–207) with fewest specs confirmed

✗ ECG claim unverified

⚠ ECG feature if genuine would be interesting, but source manufacturer unknown

Verdict: Does not meet requirements. Trading company with no manufacturing control, overpriced, specs unverified. Not recommended.

Supplier 16 — Shenzhen Zaoji Kechuang Technology Co., Ltd.

Chinese Name: 深圳市造极科创科技有限公司

Location: Guangdong, China (Shenzhen)

Platform: Alibaba.com (international)

Type: Manufacturer

Product: 2025 New — Health Smart Ring with 5ATM, magnetic charging, stainless steel, touchscreen

Category: Premium health / lifestyle smart ring

| Specification | Value |
| --- | --- |
| Price | CN¥109–135/unit |
| MOQ | 5 pieces (1 piece for some SKUs) |
| Battery | 15 mAh (sizes ≤#9), 18 mAh (sizes #10+) |
| Battery Life | 5–10 days |
| Sizes | Multiple (US standard) |
| Material | Metal / Stainless Steel (confirmed) |
| Colors | Multiple (black option available) |
| Water Resistance | 5 ATM (confirmed — explicitly stated) |
| Charging | Magnetic charging (confirmed) |
| BLE | Bluetooth LE (version references "1.5" in data — likely BLE 5.x, needs confirmation) |
| Certifications | None listed on Alibaba page |
| Touch | Touchscreen (confirmed) |
| Gesture Control | Yes — confirmed feature |
| Health Sensors | HR, SpO2, sleep, temperature, body hydration, stress, menstrual, HRV |
| Other Functions | Alarm, countdown, call reminder, message reminder, multisport tracking |
| OS Compatibility | Android and iOS |

Mira Ring Validation

✓ 5ATM water resistance confirmed

✓ Magnetic charging confirmed

✓ Stainless steel body confirmed

✓ Touchscreen confirmed

✓ Gesture control confirmed

✓ Comprehensive health sensors (HR, SpO2, temp, hydration, sleep, stress)

✓ Call reminder / message notification (useful for glasses interaction model)

✓ Low MOQ (5 pieces), Alibaba platform (easier international sourcing)

✓ Guangdong / Shenzhen manufacturer

✗ No certifications listed (FCC, CE, RoHS need to be requested)

✗ Matte black finish not explicitly confirmed

✗ BLE exact version unclear

✗ No sales volume data (newer listing)

Verdict: Best overall match. Hits every critical Mira Ring spec except certifications. Highest priority for outreach.

3. Validation Summary Table

The following table provides a comparative overview of all 16 suppliers against Mira Ring requirements. Symbols: ✓ = confirmed, ✗ = not met, ⚠ = partial/unconfirmed.

| # | Company | Price (CNY) | 5ATM | Mag Chg | BLE | Certs | Touch | SS Body | Fit |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | Huagepeng | ¥80–95 | ✗ | ✗ | ✓ | ✗ | Counter | ✗ | Low |
| 2 | Hangu Cloud | ¥45–290 | ✗ | ✗ | ✓ | ✗ | Counter | ⚠ | Low |
| 3 | Weitecheng | ¥95–110 | ✗ | ✗ | ✓ | ✗ | Button | ✗ | Low |
| 4 | Zhentongxin | ¥85–90 | ✗ | ✗ | ✓ | ✗ | Counter | ✗ | Low |
| 5 | Beiqi Info | ¥38–46 | ✗ | ✗ | ✓ | ✗ | Counter | ✗ | Low |
| 6 | Xingyao | ¥49–308 | ✗ | ✗ | 5.4⚠ | CE⚠ | Counter | ⚠ | Low |
| 7/9 | Xinyao | ¥47–63 | ✗ | ✓ | ⚠ | ✗ | ⚠ | ⚠ | Medium |
| 8 | Xinzhiyan | ¥89 | IP68⚠ | ✗ | 5.0✓ | CE+FCC✓ | ⚠ | ✓ | High |
| 10 | Huaou | ¥119 | ✓ | ⚠ | ⚠ | ✗ | ✓ | ⚠ | High |
| 11 | Xuanzhi | ¥99–105 | ⚠ | ✓ | ⚠ | ✗ | ✓ | ⚠ | High |
| 12 | Migo | ¥103 | 1ATM✗ | ✗ | ⚠ | ✗ | ✓ | ✓ | Low |
| 13 | Huagepeng (AL) | ¥75–88 | IP67⚠ | ✓ | ⚠ | FCC+CE✓ | Counter | ✗ | Medium |
| 14 | Mosslife | ¥37–41 | IPX8⚠ | ✗ | ✓ | ✗ | Counter | ✗ | Low |
| 15 | Hongsen | ¥193–207 | ✗ | ⚠ | ⚠ | ✗ | ✓ | ✗ | Low |
| 16 | Zaoji Kechuang | ¥109–135 | ✓ | ✓ | ⚠ | ✗ | ✓ | ✓ | Highest |

4. Priority Recommendations

Tier 1 — Immediate Outreach

Supplier 16 — Shenzhen Zaoji Kechuang Technology (Alibaba.com)

Best overall spec match. Confirmed: 5ATM, magnetic charging, stainless steel, touchscreen, gesture control, health sensors. Follow-up needed: FCC/CE/RoHS certifications, BLE version, matte black availability, custom firmware/SDK feasibility.

Supplier 8 — Shenzhen Xinzhiyan Medical Technology (1688.com)

Most credible manufacturer. Confirmed: IP68, BLE 5.0, CE + FCC certs, stainless steel + epoxy body, comprehensive health sensors. Follow-up needed: whether a magnetic charging variant exists or can be custom-developed, matte black option.

Tier 2 — Secondary Evaluation

Supplier 11 — Guangzhou Xuanzhi Technology (1688.com)

Best on magnetic charging + gesture control + volume pricing (¥99 at 2,000 units hits the cost target). Private label available. Follow-up needed: explicit IP/ATM waterproof rating number, certifications.

Supplier 10 — Huaou Tonghui Suzhou (1688.com)

5ATM confirmed, touchscreen confirmed. Follow-up needed: confirm magnetic charging (currently "charging case"), certifications, BLE version.

Tier 3 — Monitor / Deprioritize

Supplier 7/9 — Huizhou Xinyao Electronic (1688.com)

Highest sales volume (14,000+) and lowest price (¥47–63) — proven manufacturing scale. Magnetic charging present. However, no waterproof rating or certifications confirmed. Could be viable if waterproofing can be confirmed/upgraded.

Disqualified Suppliers

✗  Supplier 12 — 1ATM only; fails water resistance requirement

✗  Suppliers 1–6 — Counter rings; wrong product category entirely

✗  Supplier 14 — Counter ring + Type-C charging; wrong category

✗  Supplier 15 — Trading company, ABS body, no specs confirmed, overpriced